COSHH RISK ASSESSMENT

Mixed chemical bench preparation

Research Lab

| REF CAT-PW-001 | ASSESSOR David Kemp | ASSESSED 2026-05-30 | NEXT REVIEW 2027-05-30 |
| --- | --- | --- | --- |

Generated May 31, 2026 with CAT — COSHH Assessment Tool

01   OVERVIEW

| Business Unit | Research Lab |
| --- | --- |
| Location | Chemistry prep room |
| Risk Assessor | David Kemp |
| SOP Ref number(s) | SOP-CHEM-015 |
| Date of Assessment | 2026-05-30 |
| Date of Review | 2027-05-30 |
| Persons at Risk | Staff, Students |

Activity Outline

Small-scale preparation, transfer, storage and cleanup of mixed laboratory chemicals for bench work.

02   TASK HAZARDS

Hazard 1 — Glassware breakage and manual handling during chemical transfer

| How harm occurs | Cuts or dropped containers could cause injury and secondary chemical release. |
| --- | --- |
| Risk (before controls) | 9 (L3 × S3) |
| Controls in place | Use compatible trays, small containers, clear bench space and trained staff only. |
| Residual risk | 2 (L1 × S2) |
| Further action | — |
| Owner | — |
| Due | — |
| Completed | — |

03   COSHH PROCESS STEPS & CHEMICALS

Step 1 — Prepare and transfer mixed chemicals

| Engineering controls | Fume hood |
| --- | --- |
| PPE | Gloves, Goggles, Lab coat, Face shield |
| Other step controls | Use compatible secondary containment. Do not open incompatible chemicals together. Confirm glove material and any RPE need against SDS before work. |

1.1    Acetone    (CAS 67-64-1)

| Form / quantity | liquid · 250 mL |
| --- | --- |
| H-codes | H225 ; H319 ; H336 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Flammable · Harmful / irritant

1.2    Ethanol    (CAS 64-17-5)

| Form / quantity | liquid · 100 mL |
| --- | --- |
| H-codes | H225 ; H319 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Flammable · Harmful / irritant

1.3    Dichloromethane    (CAS 75-09-2)

| Form / quantity | liquid · 50 mL |
| --- | --- |
| H-codes | H315 ; H319 ; H335 ; H351 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Harmful / irritant · Health hazard

1.4    Hydrochloric acid    (CAS 7647-01-0)

| Form / quantity | liquid · 100 mL |
| --- | --- |
| H-codes | H290 ; H314 ; H335 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Corrosive · Harmful / irritant

1.5    Sulfuric acid    (CAS 7664-93-9)

| Form / quantity | liquid · 100 mL |
| --- | --- |
| H-codes | H290 ; H314 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Corrosive

1.6    Sodium hydroxide    (CAS 1310-73-2)

| Form / quantity | solid · 100 g |
| --- | --- |
| H-codes | H290 ; H314 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Corrosive

1.7    Nitric acid    (CAS 7697-37-2)

| Form / quantity | liquid · 25 mL |
| --- | --- |
| H-codes | H272 ; H290 ; H314 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Oxidising · Corrosive

1.8    Hydrogen peroxide 30%    (CAS 7722-84-1)

| Form / quantity | liquid · 50 mL |
| --- | --- |
| H-codes | H272 ; H302 ; H318 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Oxidising · Corrosive · Harmful / irritant

1.9    Sodium cyanide    (CAS 143-33-9)

| Form / quantity | solid · 10 g |
| --- | --- |
| H-codes | H300 ; H310 ; H330 ; H410 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Acute toxicity · Hazardous to environment

1.10    Phenol    (CAS 108-95-2)

| Form / quantity | solid · 25 g |
| --- | --- |
| H-codes | H301 ; H311 ; H314 ; H331 ; H341 ; H373 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Acute toxicity · Corrosive · Health hazard

1.11    Toluene    (CAS 108-88-3)

| Form / quantity | liquid · 100 mL |
| --- | --- |
| H-codes | H225 ; H304 ; H315 ; H336 ; H361 ; H373 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Flammable · Harmful / irritant · Health hazard

1.12    Formaldehyde solution    (CAS 50-00-0)

| Form / quantity | liquid · 25 mL |
| --- | --- |
| H-codes | H301 ; H311 ; H314 ; H317 ; H331 ; H341 ; H350 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Acute toxicity · Corrosive · Health hazard

1.13    Potassium permanganate    (CAS 7722-64-7)

| Form / quantity | solid · 20 g |
| --- | --- |
| H-codes | H272 ; H302 ; H410 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Oxidising · Harmful / irritant · Hazardous to environment

1.14    Sodium borohydride    (CAS 16940-66-2)

| Form / quantity | solid · 10 g |
| --- | --- |
| H-codes | H260 ; H301 ; H314 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Flammable · Acute toxicity · Corrosive

1.15    Acetonitrile    (CAS 75-05-8)

| Form / quantity | liquid · 100 mL |
| --- | --- |
| H-codes | H225 ; H302 ; H312 ; H319 ; H332 |
| WEL TWA / STEL | Check EH40/SDS / — (Manual-EH40) |
| Exposure | Up to 30 minutes per use, Weekly · routes: inhalation, skin, eye |

GHS pictograms

Flammable · Harmful / irritant

04   COSHH ESSENTIALS SCREENING

COSHH Essentials is the HSE control-banding screening tool published in HSG193. It identifies a suggested control approach for each substance from the combination of (a) the chemical hazard group derived from CLP H-statements, (b) the scale at which the substance is used, and (c) the volatility band for liquids or the dustiness band for solids. The highest approach across all substances is a screening output for assessor review, not automatic approval of controls.

It is a screening tool only. A competent assessor must verify the screening output against the SDS, task, exposure route, quantity, duration, WELs and local conditions, and may impose stricter controls. The scheme does not cover asbestos, lead, pesticides, radioactive materials or biological agents, and does not address physical hazards such as fire, explosion or reactivity.

Recommended approach — Approach 4 — Specialist advice required

Driven by Phenol — hazard group E (H341), scale small, dustiness medium, exposure predictor EP1.

Reference: HSE Generic Guidance Sheet G400 — refer to occupational hygienist / specialist.

Per-substance breakdown

| Substance | Group | H-codes | Scale | Band | EP | Approach |
| --- | --- | --- | --- | --- | --- | --- |
| Acetone | A | — | small | high (volatility) | EP2 | 1 |
| Ethanol | A | — | small | high (volatility) | EP2 | 1 |
| Dichloromethane | D | H351 | small | high (volatility) | EP2 | 3 |
| Hydrochloric acid | C | H314, H335 | small | medium (volatility) | EP2 | 2 |
| Sulfuric acid | C | H314 | small | low (volatility) | EP1 | 1 |
| Sodium hydroxide | C | H314 | small | medium (dustiness) | EP1 | 1 |
| Nitric acid | C | H314 | small | medium (volatility) | EP2 | 2 |
| Hydrogen peroxide 30% | C | H318 | small | low (volatility) | EP1 | 1 |
| Sodium cyanide | D | H300, H310, H330 | small | high (dustiness) | EP2 | 3 |
| Phenol drives controls | E | H341 | small | medium (dustiness) | EP1 | 4 |
| Toluene | D | H361 | small | medium (volatility) | EP2 | 3 |
| Formaldehyde solution drives controls | E | H341, H350 | small | high (volatility) | EP2 | 4 |
| Potassium permanganate | B | H302 | small | medium (dustiness) | EP1 | 1 |
| Sodium borohydride | C | H301, H314 | small | medium (dustiness) | EP1 | 1 |
| Acetonitrile | B | H302, H312, H332 | small | high (volatility) | EP2 | 1 |

* assumed value used because input was missing or unparseable.

Caveats & assumptions

- Group E hazard (H341) — COSHH Essentials directs to specialist advice (Approach 4). Do not rely on the banded screening.

- H304 aspiration hazard: COSHH Essentials does not fully cover aspiration risk — consider design controls against accidental swallowing/inhalation of mists.

- Group E hazard (H341, H350) — COSHH Essentials directs to specialist advice (Approach 4). Do not rely on the banded screening.

- COSHH Essentials is a screening tool. A competent risk assessor must verify the screening output against the SDS, task, exposure route, quantity, duration, WELs and local conditions, and may impose stricter controls. Not valid for asbestos, lead, pesticides, radioactive materials, or biological agents.

Reference legend

| Hazard group |  |
| --- | --- |
| Group A | Lower health hazard band: H304, H315, H319, H336 and similar lower-toxicity effects. |
| Group B | Harmful / STOT category 2 band: H302, H312, H332, H371. |
| Group C | Toxic, corrosive, serious eye damage, sensitising skin, respiratory irritation or STOT category 1/2: H301, H311, H314, H317, H318, H331, H335, H370, H373. |
| Group D | Fatal acute toxicity, suspected carcinogen / reproductive toxicant, lactation hazard or repeated-exposure organ damage: H300, H310, H330, H351, H360, H361, H362, H372. |
| Group E | Respiratory sensitiser, mutagen or carcinogen category 1 / suspected mutagen: H334, H340, H341, H350. Specialist advice is required (Approach 4). |

| Exposure predictor (EP) band |  |
| --- | --- |
| EP1 | Lowest exposure potential: small amounts with low/medium dustiness, or low-volatility millilitre-scale liquids. |
| EP2 | Low-to-moderate exposure potential: grams of high-dust solids, kg/tonne of low-dust solids, or millilitres of medium/high-volatility liquids. |
| EP3 | Moderate-to-high exposure potential: litre-scale liquid work or kilogram-scale medium/high-dust solid work. |
| EP4 | Highest exposure potential: cubic-metre-scale high-volatility liquids or tonne-scale medium/high-dust solids. |

| Control approach |  |
| --- | --- |
| Approach 1 | General ventilation and good working practice. |
| Approach 2 | Engineering control, normally local exhaust ventilation or equivalent capture / control. |
| Approach 3 | Containment or enclosure where small breaches may occur. |
| Approach 4 | Specialist advice required: the banding screen alone is not sufficient. |

GHS pictogram legend

Explosive · Flammable · Oxidising · Gas under pressure · Corrosive · Acute toxicity · Harmful / irritant · Health hazard · Hazardous to environment

05   CONTROL MEASURES

| Elimination | Each chemical was reviewed. Remove any substance not required by the SOP before work starts. |
| --- | --- |
| Substitution | Substitution must be confirmed against the method and SDS. Use lower hazard alternatives where validated and approved. |
| Reduction | Use only the minimum working quantity at the bench; return stock containers immediately after dispensing. |
| Administrative | Only trained and authorised personnel may perform the task. Current SDS for all 15 chemicals must be checked before work; reassess before changing chemicals, quantity, concentration or process. |
| Air Monitoring | Review EH40/WEL information and SDS exposure limits. Air monitoring is required where exposure cannot be demonstrated as adequately controlled by containment/LEV or after process/control changes. |
| Health Surveillance | Refer to Occupational Health where SDS, local procedure, sensitiser/CMR classification or potential exposure indicates health surveillance or medical surveillance is required. |

06   STORAGE

| Storage | Storage groups generated by CAT must be checked against SDS sections 7 and 10 before use. Keep incompatible groups segregated and retain minimum stock. |
| --- | --- |
| Incompatible substances | Segregate acids/bases, flammables/oxidisers, cyanides/acids, water-reactives/water or aqueous wastes, and toxics from incompatible groups unless SDS confirms compatibility. |

07   EMERGENCY RESPONSE AND WASTE

| Emergency — Spills | Follow SDS Section 6 and the local spill procedure. Only trained staff may tackle small known spills with suitable PPE, ventilation and compatible spill media. Escalate for vapour, toxic, reactive, oxidising, cyanide, drain, fire or uncertainty concerns. |
| --- | --- |
| Emergency — First aid | Use SDS Section 4 as definitive first-aid instruction. Move casualty away if safe, irrigate skin/eyes immediately for corrosive exposure, do not induce vomiting unless medically directed, seek urgent medical advice for toxic/CMR/reactive exposures and send SDS with casualty. |
| Emergency — Fire | Raise alarm and evacuate. Only trained staff may use extinguishers for very small incipient fires where SDS Section 5 confirms suitable media. Do not use water on water-reactive materials unless SDS/emergency responders confirm. Provide SDS and chemical inventory to responders. |
| Waste handling | Collect waste in compatible, labelled, closed containers under secondary containment. Segregate incompatible waste streams; do not dispose to drain unless SDS/local procedure explicitly permits. Use approved hazardous-waste route. |
| Other | Emergency shower, eyewash, spill kit and SDS access point must be confirmed before starting. |

08   BRIEFING RECORD

Test user    ·    2026-05-30